ELDEROS VISION

Phase 1 Product Specification

Room-Based AI Safety System for Long-Term Care

| Document Version | 1.2 |
| --- | --- |
| Classification | Confidential |
| Deployment Model | Stand-alone, on-prem edge |
| Primary Buyer | LTC / Retirement Homes |

Primary Objective: Reduce unwitnessed falls, improve response time,

create defensible incident documentation

# Document Version History

| Version | Date | Author | Changes |
| --- | --- | --- | --- |
| 0.1 | 2025-11-15 | Product Team | Initial draft — core detection capabilities, hardware architecture |
| 0.5 | 2025-12-10 | Product Team | Added software architecture, dashboard specs, privacy controls |
| 0.8 | 2026-01-20 | Product Team | Added PSW role, mobile-first interface, escalation rules |
| 1.0 | 2026-02-25 | Product Team | First release — complete Phase 1 specification |
| 1.1 | 2025-02-28 | Product Team | Added dual-sensor architecture (AI Vision + AI Sensor), 5-role RBAC |
| 1.2 | 2025-03-01 | Product Team | Added call bell integration, facility management, acceptance criteria, risk assessment |

# Table of Contents

Document Version History	1

Table of Contents	1

1. Executive Summary	1

1.1 What This Product Actually Sells	1

2. System Overview	1

2.1 Core Detection Capabilities	1

2.2 Dual-Sensor Architecture	1

2.3 Resident Mobility Profile	1

3. Hardware Architecture	1

3.1 Camera Placement Requirements	1

4. Software Architecture	1

4.1 Edge Processing Stack	1

4.2 Critical Architecture Decisions	1

5. System Health Monitoring	1

6. Alerting and Escalation	1

6.1 Alert Delivery Channels	1

6.2 Escalation Rules	1

6.3 Two-Step Response Tracking	1

7. Incident Documentation	1

7.1 Auto-Generated Incident Record	1

7.2 Post-Fall Automated Workflow	1

8. Call Bell Integration	1

8.1 Overview	1

8.2 Data Captured Per Event	1

8.3 Response Time Thresholds	1

8.4 Analytics Provided	1

8.5 Why Call Bell Data Matters for Sales	1

9. Facility Management	1

9.1 Administrative Configuration	1

9.2 Room Types	1

10. Dashboard Interfaces	1

10.1 Role Hierarchy	1

10.2 PSW View (Mobile-First)	1

10.3 Nurse View	1

10.4 Supervisor View	1

10.5 Executive View	1

10.6 Admin View	1

10.7 Navigation Structure	1

11. Additional Automation Features	1

11.1 Shift Handoff Summary	1

11.2 Weekly Facility Digest	1

11.3 Maintenance Self-Monitoring	1

12. Privacy and Compliance Controls	1

13. Data Storage and Security	1

13.1 Local Storage	1

13.2 Security Controls	1

14. Technology Stack	1

15. Implementation Phases	1

Phase 1A — Proof of Value (8 Weeks, 1 Unit / 25 Rooms)	1

Phase 1B — Full Feature Set (6 Weeks After 1A)	1

Phase 1C — Operational Maturity (4 Weeks After 1B)	1

Installation Workflow (Per Unit)	1

16. Remote Update Strategy	1

17. Staff Adoption Strategy	1

17.1 What Changes for PSWs	1

17.2 What Changes for Nurses	1

17.3 Adoption Risks and Mitigations	1

18. Acceptance Criteria	1

19. Risk Assessment	1

Appendix A: Sensor Type Summary	1

Appendix B: Permission Matrix	1

# 1. Executive Summary

ElderOS Vision is an AI-powered edge-processing safety system installed inside resident rooms in long-term care facilities. It continuously analyzes human movement and behavior patterns to detect falls, unsafe transfers, bed exits, and prolonged inactivity — without wearables and without requiring nurse documentation to operate.

The system processes video and sensor data locally on-premise and outputs events, alerts, and compliance records — not surveillance video. Phase 1 is intentionally narrow: room safety, automated documentation, proactive risk flagging, and call bell response tracking.

Operating Principle DETECT → ALERT → DOCUMENT → FOLLOW-UP

## 1.1 What This Product Actually Sells

The core value proposition is liability defense and insurance cost reduction, delivered through measurable resident safety improvements. LTC administrators purchase risk reduction with minimal staff disruption. The automated incident documentation is the primary sales differentiator — every administrator has experienced a lawsuit or near-miss where documentation was incomplete.

Phase 1 includes call bell integration as a secondary value driver — response time accountability is a persistent compliance gap in LTC, and call bell data provides immediate, measurable performance metrics without waiting for AI calibration.

Phase 1 Explicitly Excludes:

- EHR integration, vitals monitoring, medication tracking

- Staff scheduling, wearables, facility-wide RTLS tracking

- Predictive hospital analytics (deferred to Phase 2: CareIQ)

# 2. System Overview

## 2.1 Core Detection Capabilities

The system provides the following detection modules, prioritized by implementation order and reliability confidence:

| Tier | Module | Detects | Output |
| --- | --- | --- | --- |
| Tier 1 | Real-Time Fall Detection | Floor impact, uncontrolled descent, lying on floor beyond threshold | Alert within 3–6 sec with room ID, resident ID, timestamp |
| Tier 1 | Bed Exit Monitoring | Sitting on bed edge, standing from bed, night bed exit | Pre-fall intervention alerts, prolonged edge-of-bed warnings |
| Tier 2 | Unsafe Transfer Detection | Unassisted standing, wheelchair exit, instability during transfer | Real-time alert to floor staff |
| Tier 2 | Prolonged Inactivity | No movement beyond configured time, potential unconsciousness | Escalating alert based on inactivity duration |
| Tier 3 | Night Wandering | Multiple nighttime exits, pacing behavior | Pattern reporting to supervisor (not real-time) |

Deferred from Phase 1 Confusion patterns and chair slip detection are deferred. Confusion pattern analysis is a research-grade problem that will generate excessive false positives and erode staff trust. Chair slip from a ceiling-mounted wide-angle camera lacks sufficient accuracy for production use.

## 2.2 Dual-Sensor Architecture

Rooms can be equipped with one or both sensor types:

| Sensor Type | Technology | Strengths | Limitations |
| --- | --- | --- | --- |
| AI Vision | 4MP PoE IR Camera + pose estimation | Highest accuracy for fall/transfer detection, visual context | Privacy sensitivity, requires exclusion zones |
| AI Sensor | mmWave radar (60GHz) | Privacy-friendly (no video), through-blanket, no light needed | Lower spatial resolution, no visual incident context |
| Hybrid | Both AI Vision + AI Sensor | Fused confidence scoring, redundancy, best accuracy (>95%) | Higher per-room cost, more complex installation |

Each room is configured as one of three sensor types: AI Vision, AI Sensor, or Hybrid. The system UI displays the sensor type for every room on the room grid. When both sensors detect an event simultaneously, confidence scores are fused — producing higher-confidence alerts that reduce false positives. Sensor source is recorded on every event.

## 2.3 Resident Mobility Profile

The system continuously builds a behavioral baseline per resident without manual assessment forms.

Metrics Tracked:

- Walking speed and gait regularity

- Transfer stability (sit-to-stand consistency)

- Time upright per day

- Nighttime activity frequency

- Frequency of assistance-free movement

Output: Daily Fall Risk Score (0–100) with flags for rapid deterioration, increasing instability, and elevated fall probability. Risk scoring launches in observe-only mode during Phase 1A and activates in Phase 1B after baseline calibration.

# 3. Hardware Architecture

Per 25-room unit (typical deployment):

| Component | Specification | Notes |
| --- | --- | --- |
| Edge AI Appliance | NVIDIA Jetson Orin NX (8GB+) | Upgraded from Orin Nano. NX provides necessary headroom for ~$150 additional cost. |
| Operating System | Ubuntu LTS + Docker runtime | Container-based deployment for reproducible updates |
| Storage | Local SSD 1TB (encrypted) | AES-256 at rest. Self-monitoring for capacity alerts. |
| AI Vision Sensors | 4MP PoE IR, 2.8mm lens (wide FOV) | Ceiling/wall mount. Night vision required. No audio. ONVIF/RTSP. |
| AI Sensors (Radar) | 60GHz mmWave radar modules | Wall-mount 1.2–1.5m height. No privacy concerns. |
| Network | VLAN isolated, local LAN only | No cloud dependency for operation. |
| Call Bell Interface | Serial/TCP bridge | Integration with Jeron, Rauland, or Hill-Rom systems. |

Critical: Night Vision IR Quality Night vision IR quality varies wildly between camera models. Cheap IR cameras wash out at close range and lose detail at distance. Specific camera models must be tested in actual LTC rooms at night before committing to the hardware BOM.

## 3.1 Camera Placement Requirements

Camera placement is an art, not a specification line item. The deployment package must include:

- Room-type diagrams with mounting positions (standard single, standard double, corner room)

- Explicit exclusion zones (do not mount directly above bed, avoid backlight from windows)

- Field-of-view validation checklist per room

- Bathroom exclusion zone configuration

# 4. Software Architecture

## 4.1 Edge Processing Stack

| Layer | Function | Technology |
| --- | --- | --- |
| Video Ingestion | RTSP stream capture at 3–5 fps | GStreamer + NVIDIA DeepStream |
| Radar Ingestion | mmWave point cloud processing | Custom radar SDK integration |
| Person Detection | Identify and localize humans in frame | YOLOv8-nano / RT-DETR-lite (TensorRT) |
| Pose Estimation | Skeletal keypoint extraction | RTMPose-m (MMPose, TensorRT) |
| Sensor Fusion | Combine AI Vision + AI Sensor scores | Weighted fusion algorithm (hybrid rooms) |
| Event Engine | Fall detection, behavior analysis | Python state machine (custom, rule-based) |
| Alert Engine | Notifications, escalation, ack tracking | WebSocket + Twilio (SMS fallback) |
| Compliance Engine | Auto incident documentation + audit trail | FastAPI backend + PDF generation |
| Database | Event metadata, risk scores, audit logs | SQLite (single-facility, zero-config) |
| Dashboard | PSW, nurse, supervisor, executive UIs | React + FastAPI |
| System Health | Sensor heartbeat, latency, disk, NTP | Custom daemon with alert integration |
| Call Bell Interface | Nurse call system event capture | Serial/TCP middleware |

All inference is performed locally. No cloud dependency for core operation.

## 4.2 Critical Architecture Decisions

Frame Sampling

Process 3–5 fps for monitoring, burst to full frame rate on event trigger. This reduces compute load by 6–10x and allows the Orin NX to handle 25 rooms with headroom.

Rule-Based Event Engine (Not End-to-End ML)

Pose estimation uses ML. But event classification is implemented as a deterministic state machine applied to ML outputs. This makes the system debuggable, tunable per facility, and eliminates the black-box problem.

Confidence Threshold + Human Review Queue

- High confidence (>85%): Immediate alert to floor staff

- Medium confidence (60–85%): Queued for async supervisor review (no audible alarm)

- Low confidence (<60%): Logged for pattern analysis, not surfaced in real-time

Video Snippet Access Control

Short event video clips are retained for incident review. Access requires role-based authorization. In PHIPA-regulated environments, every view must be justified and recorded.

# 5. System Health Monitoring

Why This Matters If the system is down and a fall occurs, the facility is worse off than having no system — because they will be asked why their safety system was not operational. System self-monitoring is not optional.

| Monitor | Frequency | Action on Failure |
| --- | --- | --- |
| AI Vision sensor heartbeat | Every 60 seconds | Immediate alert to supervisor + dashboard flag |
| AI Sensor (radar) heartbeat | Every 60 seconds | Room marked offline if all sensors down |
| Inference pipeline latency | Continuous | Alert if latency exceeds 2x baseline |
| Disk usage | Hourly | Alert at 80%; auto-purge oldest clips at 90% |
| NTP time sync | Every 15 minutes | Alert if drift exceeds 5 seconds |
| Nightly self-test | Daily 3:00 AM | Automated diagnostic report emailed to IT |
| Call bell interface | Every 60 seconds | Alert if nurse call connection lost |

# 6. Alerting and Escalation

## 6.1 Alert Delivery Channels

- Nurse station dashboard (primary — dedicated tablet with persistent audio)

- Mobile browser on staff phones (secondary)

- SMS via Twilio (escalation fallback, facility-controlled)

Design Decision Sound alerts should come from a dedicated tablet at the nurse station, not phones. Phones get silenced. A persistent audio source at the station is more reliable for critical alerts.

## 6.2 Escalation Rules

Configurable per facility. Default escalation chain:

- PSW alerted immediately (dashboard + audio + mobile push)

- If not acknowledged within 2 minutes → nurse + supervisor alert via SMS

- If still unacknowledged within 5 minutes → charge nurse alert

- All escalation timestamps recorded in incident audit trail

## 6.3 Two-Step Response Tracking

- Acknowledge: “I see this alert” — stops escalation, records awareness time

- Resolve: “I have attended to the resident” — records physical response time

This distinction captures true response time for compliance reporting and performance analytics.

# 7. Incident Documentation

Primary Sales Feature This is the product’s most defensible sales feature. When a fall or safety event occurs, the system automatically generates an incident record without any manual documentation from staff.

## 7.1 Auto-Generated Incident Record

- Exact time of event (NTP-synchronized)

- 30-second pre-event activity summary

- Post-event resident state description

- Alert-to-acknowledgment response time

- Who acknowledged the alert (user ID + role)

- Who physically attended (resolve action timestamp)

- Repeat-fall indicator if applicable

- Sensor source (AI Vision, AI Sensor, or Fused)

## 7.2 Post-Fall Automated Workflow

- Creates incident record with full timeline

- Tags resident as high-risk on dashboard

- Schedules reassessment reminder (configurable interval)

- Flags room on supervisor dashboard

# 8. Call Bell Integration

## 8.1 Overview

ElderOS integrates with existing nurse call systems (Jeron, Rauland, Hill-Rom) to capture call bell events with precise timestamps. This is not an AI feature — it is a hardware integration that provides immediate value from day one without calibration.

Operating Principle PRESSED → RESPONDED → MEASURED

## 8.2 Data Captured Per Event

| Field | Description |
| --- | --- |
| Pressed At | Exact timestamp when resident pressed call bell (NTP-synced) |
| Responded At | Timestamp when staff physically responded/arrived |
| Response Time | Calculated: Responded At − Pressed At (seconds) |
| Responded By | Staff ID and name of responder |
| Room / Resident | Auto-linked from room and resident registry |
| Origin | Bedside, bathroom, hallway, or pendant |
| Priority | Emergency, urgent, or normal |
| Shift | Day (07:00–15:00), Evening (15:00–23:00), Night (23:00–07:00) |
| Vendor | Jeron, Rauland, or Hill-Rom (one per facility) |

## 8.3 Response Time Thresholds

| Threshold | Time | Meaning |
| --- | --- | --- |
| Good | Under 60 seconds | Staff responded promptly |
| Warning | 60–120 seconds | Acceptable but below target |
| Critical | Over 180 seconds | Compliance risk, requires review |
| Emergency Max | 60 seconds | Emergency calls must be under 60s |
| Compliance Target | 90% under 120s | Facility-wide SLA target |

## 8.4 Analytics Provided

- By Staff: Average response time, median, fastest/slowest, calls under 60s, calls over 3 minutes. Displayed as horizontal bar chart with color-coded thresholds.

- By Floor/Unit: Total calls, average response time, peak call hours, breakdown by priority and origin.

- By Shift: Day vs Evening vs Night comparison. Night shift consistently shows slower response — expected, but extreme outliers are flagged.

- 30-Day Trend: Daily call volume and average response time over time.

## 8.5 Why Call Bell Data Matters for Sales

- Measurable from day one — no AI calibration needed

- Immediately visible to administrators — clear before/after comparison

- Regulatory ammunition — Ontario LTC inspections ask about response times

- Staff accountability — individual and shift-level performance data

- Insurance-relevant — slow response after a fall call correlates with worse outcomes

This feature alone can justify a meeting with an LTC administrator who is not yet ready to discuss AI.

# 9. Facility Management

## 9.1 Administrative Configuration

The system includes an admin-only facility management interface for configuring:

- Units: Name, floor assignment (e.g., “Unit A” on Floor 1)

- Rooms: Room number, unit assignment, room type (single or semi-private), sensor type

- Cascading operations: Renaming a unit propagates to all rooms and residents. Deleting a unit removes associated rooms.

## 9.2 Room Types

| Type | Description |
| --- | --- |
| Single | One bed, one resident. Standard monitoring. |
| Semi-Private | Two beds (Zone A / Zone B), two residents. Alerts identify which bed zone triggered. |

# 10. Dashboard Interfaces

## 10.1 Role Hierarchy

ElderOS uses a 5-role RBAC model reflecting real LTC staffing:

| Role | Home Page | Primary Function |
| --- | --- | --- |
| PSW | Alerts | First responder. Receives and acknowledges alerts. Mobile-first. |
| Nurse (RPN/RN) | Alerts | Clinical oversight. PSW capabilities + incident export, shift summaries. |
| Supervisor | Alerts | Operations management. Nurse capabilities + review queue, video, call bell analytics. |
| Executive | Dashboard | Leadership reporting. Facility-wide trends, board reports. Read-only. |
| IT Admin | System Health | Technical admin. System health, settings, facility mgmt. No clinical access. |

### 10.2 PSW View (Mobile-First)

Design Principle A PSW running down a hallway should be able to acknowledge an alert with one thumb tap on a phone screen.

- Full-screen alert card on event: room number large, one-tap acknowledge

- Color-coded room grid (green / yellow / red) — glanceable from across station

- Sensor type indicator on each room tile

- No login required from facility network (session-based)

- Swipe to see resident’s recent events — no navigation menus during emergencies

- Call bell log — see active and recent call bell events

- High-risk resident list with current fall risk scores

If the PSW view requires training to use, it has failed. Zero-training operation is a hard requirement.

### 10.3 Nurse View

All PSW capabilities plus: shift summary dashboard, incident export to PDF, reports page with weekly digest and response time analytics, call bell log with response time tracking.

### 10.4 Supervisor View

All nurse capabilities plus: review queue for medium-confidence events, video clip review request workflow (access-controlled, audit-logged), resident risk ranking, call bell analytics, problem rooms heat map, one-click PDF export.

### 10.5 Executive View

Facility-wide trends: falls per month, average response time, unwitnessed fall rate. Unit-by-unit and shift-by-shift comparisons. Liability exposure indicator. Call bell analytics. Print-ready board reporting format.

### 10.6 Admin View

System health monitoring, sensor health table, facility management (units/rooms), settings (escalation rules, thresholds, retention policies, exclusion zones). No clinical data access.

## 10.7 Navigation Structure

| Nav Item | Accessible By | Description |
| --- | --- | --- |
| Alerts | PSW, Nurse, Supervisor | Real-time alert feed with acknowledge/resolve |
| Room Grid | All roles | Color-coded room status with sensor indicators |
| Review Queue | Supervisor | Medium-confidence events requiring triage |
| Incidents | PSW, Nurse, Supervisor, Executive | Historical incident log with response times |
| Residents | PSW, Nurse, Supervisor, Executive | Resident list with risk scores and fall history |
| Call Bell | PSW, Nurse, Supervisor, Executive | Call bell event log with filters |
| Call Bell Analytics | Supervisor, Executive | Response time breakdowns + trend charts |
| Shift Summary | Nurse, Supervisor | Per-shift handoff data |
| Reports | Nurse, Supervisor, Executive | Weekly digest, trends, response distribution |
| Dashboard | Executive | Facility-wide KPIs and board report |
| System Health | Admin | Sensor status, system metrics, diagnostics |
| Facility | Admin | Unit/room/floor management |
| Settings | Admin | Escalation rules, thresholds, retention |

# 11. Additional Automation Features

## 11.1 Shift Handoff Summary

Auto-generated summary at each shift change: events, unresolved alerts, residents flagged high-risk, and rooms requiring attention. Nurses will rely on this and it drives adoption.

## 11.2 Weekly Facility Digest

Auto-generated PDF emailed to administrators weekly. Contains facility-wide event counts, response time averages, risk score trends, call bell compliance rates, and system health status.

## 11.3 Maintenance Self-Monitoring

Sensor health, disk space, inference latency, and network status monitored continuously. If a sensor goes offline at 2:00 AM, the on-call contact is notified before a fall could go undetected.

# 12. Privacy and Compliance Controls

Designed for PHIPA-sensitive environments with a privacy-first architecture.

| Control | Implementation |
| --- | --- |
| No facial recognition | System identifies pose skeletons only. No biometric data captured. |
| No audio capture | Sensors selected without microphones. Audio explicitly disabled. |
| AI Sensor privacy | Radar captures point clouds only. No visual data whatsoever. |
| On-device processing | All inference performed locally. No video transmitted off-premise. |
| Video retention | Short event clips only. Configurable retention (3–14 days). Auto-purge. |
| Anonymization | Face/body silhouette rendering for demo and review scenarios. |
| Bathroom exclusion | Configurable exclusion zones prevent any processing in bathroom areas. |
| Video access control | Role-based authorization. Every access logged in audit trail. |

Recommendation Prepare a privacy whitepaper before the first sales call. Families and advocacy groups will challenge any camera-in-room system regardless of technical safeguards. Offering AI Sensor (radar-only) rooms eliminates the camera objection entirely for high-sensitivity residents.

# 13. Data Storage and Security

## 13.1 Local Storage

- Event metadata, risk scores, response timestamps, call bell events, and audit logs stored in SQLite

- Video clips: short event snippets only, retention configurable (3–14 days)

- AES-256 encryption at rest on local SSD

- Automated capacity monitoring with alerts at 80% and auto-purge at 90%

## 13.2 Security Controls

- HTTPS for all dashboard communication

- Role-based access control (PSW, nurse, supervisor, executive, IT admin)

- Complete audit logging of all user actions and data access

- No inbound external access required

- Outbound-only optional connection for remote support and updates

# 14. Technology Stack

| Component | Technology | Rationale |
| --- | --- | --- |
| Edge Compute | NVIDIA Jetson Orin NX | Sufficient GPU for 25 streams at 3–5fps. TensorRT native. |
| Person Detection | YOLOv8-nano / RT-DETR-lite | Optimized for TensorRT on Jetson. Well-documented. |
| Pose Estimation | RTMPose-m (MMPose) | Best accuracy/speed tradeoff for edge. |
| Radar Processing | Custom mmWave SDK | Point cloud processing for AI Sensor rooms. |
| Video Pipeline | GStreamer + DeepStream | NVIDIA optimized. Handles RTSP natively. |
| Event Engine | Python state machine | Debuggable, tunable per facility. Deterministic. |
| Database | SQLite | Simple, zero-config, adequate for single-facility. |
| Backend API | FastAPI (Python) | Async, lightweight, same language as ML pipeline. |
| Frontend | React + TypeScript + Tailwind | Standard, maintainable, strong mobile support. |
| State Mgmt | Zustand + React Query | Lightweight, minimal boilerplate, auto cache. |
| Charts | Recharts | Composable React chart components. |
| Alerts | WebSocket + Twilio SMS | Real-time dashboard + reliable SMS fallback. |
| Call Bell | Serial/TCP middleware | Captures events from nurse call systems. |
| Deployment | Docker Compose on Ubuntu LTS | Reproducible, updatable, standard containers. |

# 15. Implementation Phases

## Phase 1A — Proof of Value (8 Weeks, 1 Unit / 25 Rooms)

Goal: Prove the system detects falls reliably in a real environment.

- Camera + radar sensor installation + edge appliance setup

- Fall detection only (Tier 1 — no other event types yet)

- PSW dashboard with alert + one-tap acknowledge

- Auto-incident record generation

- Call bell integration active from day one — immediate measurable value

- 2-week silent monitoring for AI calibration before alert activation

Success Metrics:

- Detect falls faster than staff with <5% false positive rate per shift

- Call bell response time baseline established within first week

Critical Do not deploy all event types on day one. False positives from less-mature detections will destroy PSW trust in fall alerts. Ship falls first. Prove reliability. Then expand. Call bell tracking provides immediate wins while AI calibrates.

## Phase 1B — Full Feature Set (6 Weeks After 1A)

Goal: Expand detection capabilities and analytics after trust is established.

- Add bed exit monitoring (Tier 1)

- Add unsafe transfer detection (Tier 2)

- Add prolonged inactivity monitoring (Tier 2)

- Activate escalation chains

- Launch supervisor analytics view (including call bell analytics)

- Risk scoring baseline activated (observe-only → active)

- Nurse role enabled with shift summaries and reports access

## Phase 1C — Operational Maturity (4 Weeks After 1B)

Goal: Facility operates independently with full self-service.

- Daily risk scores active with trend alerts

- Executive dashboard launched with call bell analytics

- PDF incident report export

- Shift handoff summaries and weekly digest emails

- System health self-monitoring fully operational

- Facility management admin interface active

- Facility staff fully self-sufficient

## Installation Workflow (Per Unit)

| Week | Activity |
| --- | --- |
| Week 1 | Mount sensors, install edge appliance, connect call bell interface, configure detection/exclusion zones |
| Week 2 | Silent monitoring and calibration. AI runs but does not alert staff. Call bell tracking active (immediate value). |
| Week 3 | Activate AI alerts. Brief floor staff on dashboard (15-minute walkthrough, not formal training). |

# 16. Remote Update Strategy

“No cloud dependency” is correct for operation but creates a critical update problem. Pushing model updates, bug fixes, and configuration changes to 50+ facilities requires planning now.

Recommended Approach:

- Lightweight remote management agent (outbound-only, facility-approved)

- Docker container pull from private registry on schedule or manual trigger

- Facility IT must approve update policy during installation

- Rollback capability: previous container version retained for instant revert

- Update verification: automated self-test after update confirms pipeline is functional

Scaling Warning Without this, every software update requires a technician on-site. This becomes unsustainable beyond 10 facilities.

# 17. Staff Adoption Strategy

Key Insight Staff resistance will kill this product faster than any technical issue. The spec must address what changes for PSWs and nurses explicitly.

## 17.1 What Changes for PSWs

- Added: Audio/visual alerts on event. One-tap acknowledge. Call bell response tracking.

- Removed: Manual incident report writing. Pressure mat and chair alarm management.

- Unchanged: All clinical assessment, direct care, and treatment decisions remain with nursing staff.

## 17.2 What Changes for Nurses

- Added: Shift summaries auto-generated. Incident export with complete timelines. Call bell oversight.

- Removed: Manual documentation of fall events. Manual compilation of shift handoff notes.

- Unchanged: Clinical decision-making, medication management, care planning.

## 17.3 Adoption Risks and Mitigations

- “Big Brother” perception: Address directly during installation. Show anonymized silhouette mode. Offer AI Sensor (radar-only) rooms. Frame call bell tracking as workload distribution support.

- Alert fatigue: Managed through confidence tiers and per-facility tuning. Phased rollout prevents overload.

- “More work” perception: Demonstrate one-tap acknowledgment. Auto-documentation removes paperwork. Net reduction in workload.

- Call bell accountability resistance: Frame as facility improvement, not surveillance. Show aggregate data first. Allow opt-in period before naming individuals.

# 18. Acceptance Criteria

The product is considered successful when a facility can demonstrate all five outcomes:

| # | Outcome | Measurement | Target |
| --- | --- | --- | --- |
| 1 | Faster fall response | Alert-to-attendance time | <3 minutes average |
| 2 | Reduce unwitnessed falls | Falls detected before staff discovery | >80% detected by system |
| 3 | Complete documentation | Events with full auto audit trail | 100% of detected events |
| 4 | Ongoing risk monitoring | Daily risk scores generated | All residents by Phase 1C |
| 5 | Call bell compliance | Calls answered within 2 minutes | >90% facility-wide |

If these five outcomes are achieved, the facility has measurable risk reduction — which is what they actually purchase.

# 19. Risk Assessment

| Risk | Severity | Mitigation |
| --- | --- | --- |
| False positive alert fatigue | Critical | Confidence tiers, phased rollout, per-facility tuning. 5 false alarms/shift = staff ignore all. |
| Compute capacity on edge device | High | Orin NX (not Nano). Frame sampling 3–5fps. Monitor utilization from day one. |
| Camera placement / occlusion | High | Room-type installation playbook. FOV validation checklist per room. |
| Night vision IR quality | High | Test camera models in actual LTC rooms at night before committing to BOM. |
| Privacy / advocacy pushback | Medium | Privacy whitepaper ready before first sales call. Anonymized demo. AI Sensor option. |
| Software update at scale | Medium | Design remote update agent now. Build in Phase 1B. On-site unsustainable at scale. |
| Support burden at scale | Medium | Remote monitoring of every deployed unit essential by facility #10. |
| Staff adoption resistance | Medium | Clear “what changes” per role. Show net workload reduction. Zero-training interface. |
| Call bell integration compat. | Medium | Jeron, Rauland, Hill-Rom cover >80% installed base. Test before each deployment. |
| Call bell tracking resistance | Low | Frame as facility improvement. Show aggregate data first. Opt-in period. |

# Appendix A: Sensor Type Summary

| Configuration | Per-Room Hardware | Detection Quality | Privacy Level | Cost |
| --- | --- | --- | --- | --- |
| AI Vision Only | 1x 4MP PoE IR camera | Highest (visual + pose) | Medium (video local) | $ |
| AI Sensor Only | 1x 60GHz mmWave radar | Good (motion + position) | Highest (no video) | $ |
| Hybrid | 1x camera + 1x radar | Best (fused confidence) | Medium (same as Vision) | $$ |

# Appendix B: Permission Matrix

| Permission | PSW | Nurse | Supervisor | Executive | Admin |
| --- | --- | --- | --- | --- | --- |
| View/ack/resolve alerts | Yes | Yes | Yes | — | View only |
| View room grid | Yes | Yes | Yes | Yes | Yes |
| View incidents | Yes | Yes | Yes | Yes | Yes |
| Export incidents | — | Yes | Yes | Yes | — |
| Review queue triage | — | — | Yes | — | — |
| Request/view clips | — | — | Yes | — | — |
| View residents | Yes | Yes | Yes | Yes | — |
| View call bell log | Yes | Yes | Yes | Yes | — |
| Respond to call bells | Yes | Yes | Yes | — | — |
| View call bell analytics | — | — | Yes | Yes | — |
| View shift summary | — | Yes | Yes | — | — |
| View reports | — | Yes | Yes | Yes | — |
| View board report | — | — | — | Yes | — |
| View system health | — | — | — | — | Yes |
| Manage settings | — | — | — | — | Yes |
| Manage facility | — | — | — | — | Yes |

Document Version 1.2 — Updated to reflect dual-sensor architecture, 5-role RBAC, call bell integration, and facility management capabilities.